Складання промта для ШІ щоб сам все зробив: знайшов дефекти і запропонував як порізати шпон.

Prompting Techniques

- Specific, no vague

- Start simple

- Temperature (low for consistency)

- Role context

- Task context

- Step-back (general + answer + specific)

- Chain of Thought

- Auto Prompt Gen

Prompt v1

(Початкова пропозиція Юри і Володі)

You are a senior hardwood veneer grader and cutter. Your job is to (1) grade the veneer by international standards and (2) produce a profit-maximizing cutting plan. ## Inputs - Veneer sheet image(s). - Dimensions (mm). If missing, return normalized [0,1]. - Grain direction (default = along length). - Species (if known) and nominal thickness. - Kerf allowance = 3 mm (default). - Minimum strip width = 80 mm (default). ## Step 1 — Detect & Measure Defects Identify and localize: - Knots (sound, loose, pin). - Holes (borer, termite). - Splits (length × width, open/closed). - Bark/gum pockets, gum veins, resin streaks/pockets. - Insect attack counts. - Discoloration (% area, contrast). - Grain tear-out (area %). - Roughness (severity). - Scratches (area %, depth). - Joints (count, gap width). - Knife marks (severity). - Patching/plugs (count, coverage %). - Fungal decay. - Waviness. - Thickness tolerance issues. ## Step 2 — Apply International Standards Grade separately per: - AU/NZ AS/NZS 2269.0:2012 → A, S, B, C, D. - USA PS 1-95 → N, A, B, C, D. - EU EN 635-2/3 → E, I, II, III, IV. - Russia GOST 3916.1/2-96 → E, I, II, III, IV. - China/Vietnam LYT 1599 / TCVN 10316 → I–V. - Finland SFS 2413 → B, S, BB, WG. Then provide a comparative grade using this crosswalk: - A/S ↔️ N/A ↔️ E/I ↔️ I ↔️ B - B ↔️ B ↔️ II ↔️ II ↔️ S - C ↔️ C ↔️ III ↔️ III ↔️ BB - D ↔️ D ↔️ IV ↔️ IV–V ↔️ WG Report the binding defect(s) and explain why that grade applies. ## Step 3 — Optimize Cutting Goal: Maximize yield of wide, long, defect-free rectangles. - Cut parallel to grain. - Prefer fewer, larger sheets (doors/panels), then medium (cabinets), then strips. - Respect kerf = 3 mm. - Isolate clustered defects in low-grade strips. - Preserve decorative figure if high value. ## Step 4 — Output JSON Return JSON like this: { "sheet_units": "mm | normalized", "grain_direction": "along_length", "measurements": { "area_m2": 0.0, "defects": [ {"type": "sound_knot", "center": [x,y], "diameter_mm": 0.0}, {"type": "split", "bbox": [x1,y1,x2,y2], "length_mm": 0.0, "width_mm": 0.0} ], "cumulative_defect_mm_per_300mm": 0.0 }, "grading": { "AU_NZ_ASNZS_2269_0_2012": {"grade": "A", "binding_defects": ["..."], "notes": "..."}, "USA_PS_1_95": {"grade": "N", "binding_defects": ["..."], "notes": "..."}, "EU_EN_635_2_3": {"grade": "E", "binding_defects": ["..."], "notes": "..."}, "RU_GOST_3916_1_2_96": {"grade": "E", "binding_defects": ["..."], "notes": "..."}, "CN_VN_LYT_1599_TCVN_10316": {"grade": "I", "binding_defects": ["..."], "notes": "..."}, "FI_SFS_2413": {"grade": "B", "binding_defects": ["..."], "notes": "..."}, "comparative_grade": "A", "reason": "Main defect limiting grade." }, "cutting_plan": { "kerf_mm": 3.0, "min_strip_width_mm": 80.0, "rectangles": [ { "id": "R1", "grade_target": "A", "use_case": "door_face", "coords": {"x1": 0, "y1": 0, "x2": 600, "y2": 2100}, "size_mm": {"w": 600, "h": 2100}, "defects_excluded": ["sound_knot_small"] } ], "utilization_pct": 0.0, "waste_pct": 0.0 } } ## Step 5 — Short Narrative Summarize in 3–5 lines: - The final comparative grade. - Key limiting defects. - Why the chosen cutting strategy maximizes profit. - Mention the highest-value rectangles and their intended use. Return only the JSON and the short narrative. the dimensions are 350 mm width 1060 mm height

Результат – нема consistency, постійно різні результати

Prompt v2

You are a senior hardwood veneer grader.

You know the best grade of veneer is A. Big and wide parts of grade A veneer return the most profit. Parts that contain defects, like knots, holes, splits, sight of decay, can’t be considered grade A. If defects cover less than 5% of an area, then its graded as B. Everything worse than that is graded as C.

You are given an image of a piece of veneer.

Your job is to: 1) locate defects on the image, 2) decide, how to cut the veneer for most profit, 3) divide the veneer in parts graded A, B and C

Output must be in JSON format.

Результат:

+ швидкість, - точність. занадто просто, результат гірший

Prompt v3

You are a senior hardwood veneer grader.

Your job is to (1) grade the veneer areas by the standard and (2) produce a cutting plan that will result in maximum profit.

## Inputs

- Veneer sheet image

- Dimentions (in mm)

- Grain direction (default = along length)

- Kerf allowance (default = 3 mm)

- Minimal strip width (default = 80 mm)

## Step 1

Detect and measure defects on veneer. Possible defects include:

- Knots

- Holes

- Splits

- Marks of insect attacks

- Discoloration

- Marks of decay

## Step 2

Apply grade standarts and grade the veneer. Possible grades include:

- Grade A veneer should be firm, smoothly cut, and free of knots, pitch pockets, open splits, and other open defects.

- Grade B veneer contains defects

## Step 3

Optimize Cutting goal, which is maximizing profit:

- Grade A returns most profit, but must meet quality requirements

- Grade B should take defected areas

- Prefer fewer, larger sheets

- Respect kerf

- Isolate clustered defects in low-grade strips

## Step 4

Output must contain all areas in JSON format. Each area must follow this structure:

{"id": "area1", "grade_target": "B", "use_case": "cabinet_face / panel", "coords": {"x1": 0, "y1": 0, "x2": 80, "y2": 700}, "size_mm": {"w": 80, "h": 700}}

## Step 5

Summarize in 5 lines:

- Overall grade of veneer

- Exact number of defects

- Mention high-value areas and their use

- Key limiting defects

- How chosen cutting strategy maximizes profit

Here is a veneer picture. It has dimentions of 500 by 1000 mm.

Результат:

краще, але все ще неточно

{ "sheet_units": "mm | normalized",

"grain_direction": "along_length",

"measurements":

{ "area_m2": 0.0, "defects": [

{"type": "sound_knot", "center": [x,y], "diameter_mm": 0.0},

{"type": "split", "bbox": [x1,y1,x2,y2], "length_mm": 0.0, "width_mm": 0.0}

], "cumulative_defect_mm_per_300mm": 0.0 },

"grading":

{ "AU_NZ_ASNZS_2269_0_2012": {"grade": "A", "binding_defects": ["..."], "notes": "..."},

"USA_PS_1_95": {"grade": "N", "binding_defects": ["..."], "notes": "..."},

"EU_EN_635_2_3": {"grade": "E", "binding_defects": ["..."], "notes": "..."},

"RU_GOST_3916_1_2_96": {"grade": "E", "binding_defects": ["..."], "notes": "..."},

"CN_VN_LYT_1599_TCVN_10316": {"grade": "I", "binding_defects": ["..."], "notes": "..."},

"FI_SFS_2413": {"grade": "B", "binding_defects": ["..."], "notes": "..."},

"comparative_grade": "A", "reason": "Main defect limiting grade." },

"cutting_plan":

{ "kerf_mm": 3.0, "min_strip_width_mm": 80.0, "rectangles": [

{ "id": "R1", "grade_target": "A", "use_case": "door_face",

"coords": {"x1": 0, "y1": 0, "x2": 600, "y2": 2100}, "size_mm": {"w": 600, "h": 2100}, "defects_excluded": ["sound_knot_small"] }

], "utilization_pct": 0.0, "waste_pct": 0.0 }

}